# General

## General

### Support function-based needs and workspace allocation that reduce renovation and reconfiguration costs.

### Promote the safety, health, and wellness of building users.

### Fire taping products are not to be used, in either new construction or renovations. Putty, caulking, spray, or other fire stopping joints are acceptable.

# Products

## Headwalls

### Headwalls will be designed so that raising/lowering of the bed or stretchers will not catch/interfere the headwall or adjacent equipment and cause damage to either one.

### Provide multiple rails system for the installation of headwall accessories and the storage of a small quantity of medical surgical supplies for ease of access for direct Patient care.

### Provide all rails, accessories, and backing required for mounting monitors, baskets and other equipment as required.

### Headwalls to provide the following:

#### Non-institutional and modern design elements including finishes and colours that are coordinated with the interior design concept.

#### Services outlets, lighting and lighting controls as required.

#### Fibre reinforced laminate (FRL):

##### Wood-grain fibre reinforced laminate as specified herein, to all exposed surfaces or has wood-look components in prefabricated system

##### 1.9 mm (0.075 inches) thick, monolithic panel with 20% continuous glass fibres; with the following properties:

###### Barcol Hardness: ASTM D2583, 35 typical

###### Wear Resistance: NEMA LD3: Minimum 3,500 cycles

###### Surface Burning: ASTM E84, Class A. Flame Spread: 25 or less. Smoke Developed: 30 or less.

### Patient Rooms and Exam/Treatment Rooms will be provided with double headwall medical gas panels on each side of Patient bed.

# Execution

### Not Used

END OF SECTION